Industrial Computer

QY-U310

- Products Features:

- Industrial-Grade Industrial Computer

- Support Mini-ITX Mother Board

- Intel 4/6/7/8/9/10/12/13/14th-i3/i5/i7/i9 desktop CPU

- Standard 1U ATX power, 300w to 1300w

- More expansion slots, Mini PCIE / M.2 / PCIE

- Support remote power on and off

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-U310

- 1-1.B81 Mother Board Specification:

| Mother Board Model | QY-B81AH-H |
| --- | --- |
| Chipset | Intel B81, Support Intel Core 4th-i3/i5/i7 / Pentium / Celeron Desktop CPU, LGA 1150 |
| Memory | Support DDR3-1333/1600 MHz, 2*non-ECC U-DIMM Slots, Up to 16GB |
| Storage | 1*MSATA SSD slot 1*SATA 3.0 SSD slot 1*SATA 2.0 SSD slot |
| Display | 1*HDMI: Resolution max up to 3840*2160@30hz 1*VGA: Resolution max up to 1920*1200@60Hz |
| Expansion | 1*PCIE 16x slot 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Realtek 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 6*USB 3.0 (Type-A) 10*USB 2.0 (Header) |
| COM | 2*RS-232 (DB9 Type) 1*RS-232 / 485 (Box) 3*RS-232 (Box) 4*RS-232 (Header) |
| Audio | 1*Line-out+1*Line-in+MIC |
| Mother Board Type | Mini-ITX |

QY-U310

- 1-2.H110 Mother Board Specification:

| Mother Board Model | QY-B11AH-A |
| --- | --- |
| Chipset | Intel H110, Support Intel Core 6/7/8/9th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1151 |
| Memory | Support DDR4-1866/2133 MHz, 2*non-ECC U-DIMM Slots, Up to 32GB |
| Storage | 1*MSATA SSD slot 2*SATA3.0 SSD slots |
| Display | 1*HDMI: Resolution max up to 4096*2160@24Hz 1*VGA: Resolution max up to 1920*1200@60Hz |
| Expansion | 1*PCIE 16x slot 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Realtek GbE LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 8*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) 6*USB 2.0 (Header) |
| COM | 2*RS-232 (DB9 Type) 1*RS-232 / 485 (Header) 7*RS-232 (Header) |
| Audio | 1*Line-out+1*MIC |
| Mother Board Type | Mini-ITX |

| Mother Board Model | QY-H472A-X |
| --- | --- |
| Chipset | Intel H470 Support Intel Core 10th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1200 |
| Memory | Support DDR4-2133/2400/2666/2933 MHz, 2*U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 SATA SSD slot 1*SATA3.0 SSD slot |
| Display | 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 1*PCIE 16x slot 1*M.2 Key-E slot, support WIFI module |
| Ethernet | 2*Realtek GbE LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 6*USB 3.0 (Type-A) 4*USB 2.0 (Type-A) 2*USB 2.0 (Header) |
| COM | 10*RS-232 (Header) |
| Audio | 1*Line-out+1*MIC |
| Mother Board Type | Mini-ITX |

QY-U310

- 1-3.H470 Mother Board Specification:

| Mother Board Model | QY-12S2A-H610-X |
| --- | --- |
| Chipset | Intel H610, Support Intel Core 12/13/14th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1700 |
| Memory | Support DDR4-3200 MHz, 2*non-ECC U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 NVME SSD slot 1*SATA 3.0 SSD slot |
| Display | 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 1*PCIE 16x slot 1*M.2 Key-E slot, Support WIFI module |
| Ethernet | 2*Realtek RTL8111H GbE LAN Chip (10 / 100 / 1000 /2500 Mbps, RJ-45 Type) |
| USB | 4*USB 3.2 (Type-A) 6*USB 2.0 (Type-A) 2*USB 2.0 (Header) |
| COM | 10*RS-232 (Header) |
| Audio | 1*Line-out+1*MIC |
| Mother Board Type | Mini-ITX |

QY-U310

- 1-4.H610 Mother Board Specification:

QY-U310

2.Device Specification:

| Power Input | 1U ATX Power Module (300w / 500w / 800w / 1000w / 1300w) |
| --- | --- |
|  | Support Auto-Start |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fan Design |
| Installation | Desktop / Embedded |
| Size | 270mm*226.5mm*167mm |
| System | Windows and Linux |
| Mother Board Type | Mini-ITX |

QY-U310

3.Ordering Information:

| Model | Chipset | CPU | LAN | USB | COM | RAM | Display | SSD | Expansion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-U310 | B81 | 4th | 2 | 6*3.0 10*2.0 | 1*RS-232/485 9*RS-232 | 2*DDR 3 Up to 16GB | 1*HDMI 1*VGA | 1*MSATA 2*SATA | 1*Mini PCIE 1*PCIE 16x |
|  | H110 | 6th/7th/8th/9th |  | 8*3.0 8*2.0 | 1*RS-232/485 9*RS-232 | 2*DDR 4 Up to 32GB | 1*VGA 1*HDMI | 1*MSATA 2*SATA | 1*Mini PCIE 1*PCIE 16x |
|  | H470 | 10th |  | 6*3.0 6*2.0 | 10*RS-232 | 2*DDR 4 Up to 64GB | 1*VGA 1*HDMI | 1*M.2 SATA 1*SATA | 1*M.2 1*PCIE 16x |
|  | H610 | 12th/13th/14th |  | 4*3.2 8*2.0 | 10*RS-232 | 2*DDR 4 Up to 64GB | 1*HDMI 1*VGA | 1*M.2 PCIE 1*SATA | 1*M.2 1*PCIE 16x |

4.System Information:

| CPU | System |
| --- | --- |
| 4th / 6th / 7th | Windows 7 / 10 and Linux |
| 8th / 9th / 10th / 12th / 13th / 14th | Windows 10 / 11 and Linux |

5.Fittings Information:

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| SSD | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | M.2 NVME / SATA | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| GSM Module | 4G | 4G+GPS |
| WIFI Module | 7260AC / AX210 |  |
| ATX Power | 300w / 500w / 800w / 1000w / 1300w |  |

QY-U310

6.Package List:

| - Power Cable | 1*Power Cable |
| --- | --- |

QY-U310

7.Dimensions: