Industrial Computer

QY-U350

- Products Features:

- Industrial-Grade Industrial Computer

- Support ATX / Micro-ATX / Mini-ITX Mother Board

- Intel 4/6/7/8/9/10/11/12/13/14th-i3/i5/i7/i9 desktop CPU

- Standard ATX power, 300w to 1300w

- More expansion slots, Mini PCIE / M.2 / PCIE / PCI

- Support remote power on and off

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-U350

- 1-1.H81 Mother Board Specification:

| Mother Board Model | QY-H8134-X |
| --- | --- |
| Chipset | Intel H81, Support Intel Core 4th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1150 |
| Memory | Support DDR3-1333 / 1600 MHz, 2*DIMM Slots, Up to 16GB |
| Storage | 1*MSATA SSD slot 1*SATA 3.0+2*SATA 2.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz |
| Expansion | 1*PCIE 16x slot+1*PCIE 4x slot+1*PCIE 1x slot 4*PCI slots |
| Ethernet | 2*Intel I210 GbE LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 2*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) 6*USB 2.0 (Header) |
| COM | 1*RS-232 / 485 (DB9 type) 5*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+MIC |
| Mother Board Type | ATX |

QY-U350

- 1-2.B85 Mother Board Specification:

| Mother Board Model | QY-B85-ATX-V11 |
| --- | --- |
| Chipset | Intel B85, Support Intel Core 4th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1150 |
| Memory | Support DDR3-1333 / 1600 MHz, 2*SO-DIMM Slots, Up to 16GB |
| Storage | 1*MSATA SSD slot 4*SATA 3.0 slots |
| Display | 1*DP: Resolution max up to 2560*1440@60Hz 1*HDMI: Resolution max up to 2560*1440@60Hz 1*VGA: Resolution max up to 1920*1080@60Hz |
| Expansion | 1*PCIE 16x slot+2*PCIE 4x slots 2*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 3*USB 2.0 (Type-A) 8*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*MIC |
| Mother Board Type | Micro-ATX |

| Mother Board Model | QY-F1162-A-H |
| --- | --- |
| Chipset | Intel H110, Support Intel Core 6/7/8/9th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1151 |
| Memory | Support DDR4-2133/2400/2600 MHz, 2*U-DIMM Slots, Up to 128GB |
| Storage | 1*MSATA SSD slot 4*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up tp 1920*1080@60Hz |
| Expansion | 1*PCIE 16x slot+1*PCIE 4x slot+1*PCIE 1x slot 4*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel (i210V+i219V) LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) 3*USB 2.0 (Header) |
| COM | 2*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

- 1-3.H110 Mother Board Specification:

QY-U350

| Mother Board Model | QY-Z3932-X |
| --- | --- |
| Chipset | Intel Z390, Support Intel Core 6/8/9th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1151 |
| Memory | Support DDR4-2666 MHz, 2*Non-ECC U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 NVME/SATA SSD slot 4*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 14096*2160@30Hz |
| Expansion | 1*PCIE 16x slot+1*PCIE 4x slot 2*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel (i219V+i225V) LAN Chip (10 / 100 / 1000 / 2500 Mbps, RJ-45 Type) |
| USB | 4*USB 3.2 (Type-A) 2*USB 2.0 (Type-A) 5*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | Micro-ATX |

QY-U350

- 1-4.Z390 Mother Board Specification:

| Mother Board Model | QY-F1762-H |
| --- | --- |
| Chipset | Intel Q170, Support Intel Core 6/7/8/9th-i3/i5/i7 / Pentium / Celeron desktop CPU, LGA 1151 |
| Memory | Support DDR4-2133/2400/2600 MHz, 4*U-DIMM Slots, Up to128GB |
| Storage | 1*MSATA SSD slot 3*SATA 3.0 slots 1*M.2 NVME SSD slot |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 3840*2160@30Hz |
| Expansion | 2*PCIE 16x slots+2*PCIE 4x slots+1*PCIE 1x slot 2*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 6*USB 3.0 (Type-A) 8*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

- 1-5. Q170 Mother Board Specification:

QY-U350

| Mother Board Model | QY-H4734-X |
| --- | --- |
| Chipset | Intel H470, Support Intel Core 10th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1200 |
| Memory | Support DDR4-2133/2400/2666/2933 MHz, 2*U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 SATA SSD slot 3*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 1*PCIE 16x slot+2*PCIE 4x slots 4*PCI slots 1*M.2 Key-E slot, support 4G and WIFI module |
| Ethernet | 2*Intel (i219V+225V) LAN Chip (10 / 100 / 1000 / 2500 Mbps, RJ-45 Type) |
| USB | 6*USB 3.0 (Type-A) 2*USB 3.0 (Header) 5*USB 2.0 (Header) |
| COM | 2*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

QY-U350

- 1-6.H470 Mother Board Specification:

QY-U350

- 1-7.Q470 Mother Board Specification:

| Mother Board Model | QY-Q4752A-X |
| --- | --- |
| Chipset | Intel Q470, Support Intel Core 10/11th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1200 |
| Memory | Support DDR4-2400/2600/2933 MHz, 4*Non-ECC U-DIMM Slots, Up to 128GB |
| Storage | 1*M.2 NVME / SATA SSD slot 4*SATA 3.0 slots, Support RST RAID 0,1,5,10 |
| Display | 1*DVI-D: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 2*PCIE 16x slots+3*PCIE 4x slots 2*PCI slots 1*Mini PCIE slot, support WIFI and 4G module |
| Ethernet | 2*Intel (I219+I211) 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 2*USB 3.0 (Header) 2*USB 2.0 (Type-A) 5*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

- 1-8.H610-DDR 4 Mother Board Specification:

| Mother Board Model | QY-F6162-A-H-DDR 4 |
| --- | --- |
| Chipset | Intel H610, Support Intel Core 12/13/14th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1700 |
| Memory | Support DDR4-3200 MHz, 2*U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 SATA SSD slot 3*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 1*PCIE 16x slot+3*PCIE 4x slots 3*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel (i219+i255) LAN Chip (10 / 100 / 1000 / 2500 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) 4*USB 2.0 (Header) |
| COM | 2*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

QY-U350

| Mother Board Model | QY-F6162-A-H-DDR 5 |
| --- | --- |
| Chipset | Intel H610, Support Intel Core 12/13/14th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1700 |
| Memory | Support DDR5-4800 MHz, 2*U-DIMM Slots, Up to 64GB |
| Storage | 1*M.2 SATA SSD slot 4*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*DP: Resolution max up to 7680*4320@60Hz 1*HDMI: Resolution max up to 4096*2160@30Hz |
| Expansion | 1*PCIE 16x slot+2*PCIE 4x slots 4*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 7*USB 2.0 (Type-A) 2*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

QY-U350

- 1-9.H610-DDR 5 Mother Board Specification:

| Mother Board Model | QY-F6762-A-H |
| --- | --- |
| Chipset | Intel Q670, Support Intel Core 12/13/14th-i3/i5/i7/i9 / Pentium / Celeron desktop CPU, LGA 1700 |
| Memory | Support DDR5-4800 MHz, 4*U-DIMM Slots, Up to 128GB |
| Storage | 1*M.2 NVME SSD slot 4*SATA 3.0 slots |
| Display | 1*DVI: Resolution max up to 1920*1200@60Hz 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@24Hz 1*DP: Resolution max up to 7680*4320@60hz |
| Expansion | 2*PCIE 16x slots+3*PCIE 4x slots 2*PCI slots 1*Mini PCIE slot, support 4G and WIFI module |
| Ethernet | 2*Intel 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 8*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) 2*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

QY-U350

- 1-10.Q670 Mother Board Specification:

QY-U350

- 1-11.Q870 Mother Board Specification:

| Mother Board Model | QY-Q8762-X |
| --- | --- |
| Chipset | Intel Q870, Support Intel Core Ultra (Series-2) desktop CPU, LGA 1851 |
| Memory | Support DDR5-4800 MHz, 4*U-DIMM Slots, Up to 192GB |
| Storage | 1*M.2 NVME SSD slot 4*SATA 3.0 slots |
| Display | 1*VGA: Resolution max up to 1920*1200@60Hz 1*HDMI: Resolution max up to 4096*2160@24Hz 1*DP: Resolution max up to 4096*2106@60hz |
| Expansion | 2*PCIE 16x slots+4*PCIE 4x slots 1*PCI slot 1*M.2 Key-E slot, support WIFI module |
| Ethernet | 2*Intel 1Gbps LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.2 Gen1 (Type-A) 2*USB 3.2 Gen2 (Type-A) 7*USB 2.0 (Header) |
| COM | 1*RS-232 / 422 / 485 (DB9 type) 1*RS-232 / 485 (DB9 type) 4*RS-232 (DB9 type) |
| Audio | 1*Line-out+1*Line-in+1*MIC |
| Mother Board Type | ATX |

QY-U350

2.Device Specification:

| Power Input | ATX Power or Redundant Power Module (300w / 500w / 800w / 1000w / 1300w) |
| --- | --- |
|  | Support Auto-Start |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fan Design |
| Installation | Desktop / Embedded |
| Size | 375mm*325.2mm*175mm |
| System | Windows and Linux |
| Mother Board Type | Support ATX / Micro-ATX / Mini-ITX |
| GPU | Support |

QY-U350

3.Ordering Information:

Note：Please contact our sales staff if you require other chipsets.

| Model | Chipset | CPU | LAN | USB | COM | RAM | Display | SSD | Expansion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-U350 | H81 | 4th | 2 | 2*3.0 8*2.0 | 1*RS-232/485 5*RS-232 | 2*DDR 3 Up to 16GB | 1*DVI 1*VGA | 1*MSATA 3*SATA | 1*Mini PCIE 1*PCIE 16x 1*PCIE 4x 1*PCIE 1x 4*PCI |
|  | B85 | 4th |  | 4*3.0 7*2.0 | 2*RS-232/422/485 4*RS-232 | 2*DDR 3 Up to 16GB | 1*VGA 1*HDMI 1*DP | 1*MSATA 4*SATA | 1*Mini PCIE 1*PCIE 16x 1*PCIE 4x 2*PCI |
|  | H110 | 6th/7th/8th/9th |  | 4*3.0 11*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 2*DDR 4 Up to 128GB | 1*VGA 1*DVI 1*HDMI | 1*MSATA 4*SATA | 1*Mini PCIE 1*PCIE 16x 2*PCIE 4x 4*PCI |
|  | Z390 | 6th/8th/9th |  | 4*3.2 7*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 2*DDR 4 Up to 64GB | 1*VGA 1*DVI 1*HDMI | 1*M.2 4*SATA | 1*Mini PCIE 1*PCIE 16x 1*PCIE 4x 2*PCI |
|  | Q170 | 6th/7th/8th/9th |  | 6*3.0 8*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 4*DDR 4 Up to 128GB | 1*DVI 1*HDMI 1*VGA | 1*M.2 1*MSATA 4*SATA | 1*Mini PCIE 2*PCIE 16x 3*PCIE 4x 2*PCI |
| QY-U350 | H470 | 10th | 2 | 8*3.0 5*2.0 | 2*RS-232/485 4*RS-232 | 2*DDR 4 Up to 64GB | 1*DVI 1*HDMI 1*VGA | 1*M.2 3*SATA | 1*M.2 1*PCIE 16x 1*PCIE 4x 4*PCI |
|  | Q470 | 10th/11th |  | 6*3.0 7*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 4*DDR 4 Up to 128GB | 1*DVI 1*HDMI 1*VGA | 1*M.2 4*SATA | 1*Mini PCIE 2*PCIE 16x 3*PCIE 4x 2*PCI |
|  | H610 DDR 4 | 12th/13th/14th |  | 4*3.0 7*2.0 | 2*RS-232/485 4*RS-232 | 2*DDR 4 Up to 64GB | 1*DVI 1*HDMI 1*VGA | 1*M.2 3*SATA | 1*Mini PCIE 1*PCIE 16x 3*PCIE 4x 3*PCI |
|  | H610 DDR 5 | 12th/13th/14th |  | 4*3.0 9*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 2*DDR 5 Up to 64GB | 1*DP 1*HDMI 1*VGA | 1*M.2 4*SATA | 1*Mini PCIE 1*PCIE 16x 2*PCIE 4x 4*PCI |
|  | Q670 | 12th/13th/14th |  | 8*3.0 5*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 4*DDR 5 Up to 128GB | 1*DVI 1*HDMI 1*VGA 1*DP | 1*M.2 4*SATA | 1*Mini PCIE 2*PCIE 16x 3*PCIE 4x 2*PCI |
|  | Q870 | Ultra 2 |  | 6*3.2 7*2.0 | 1*RS-232/422/485 1*RS-232/485 4*RS-232 | 4*DDR 5 Up to 192GB | 1*HDMI 1*VGA 1*DP | 1*M.2 4*SATA | 1*M.2 2*PCIE 16x 4*PCIE 4x 1*PCI |

QY-U350

4.System Information:

QY-U350

| CPU | System |
| --- | --- |
| 4th / 6th / 7th | Windows 7 / 10 and Linux |
| 8th / 9th / 10th / 11th 12th / 13th / 14th / Ultra 2 | Windows 10 / 11 and Linux |

5.Fittings Information:

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
|  | DDR 5 | 4GB / 8GB / 16GB / 32GB / 64GB |
| SSD | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | M.2 NVME / SATA | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| WIFI Module | 7260AC / AX210 |  |
| ATX Power | 300w / 500w / 800w / 1000w / 1300w |  |

6.Package List:

| - Power Cable | 1*Power Cable |
| --- | --- |

QY-U350

7.Dimensions: