Industrial Mini PC

QY-B5000

High Performance Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Core 13th-i5 / i7 CPU

- 2*LAN, 6*USB, 6*COM, 1*HDMI, 1*VGA, 14*GPIO

- 2*Mini PCIE Expansion Slots, Support 4G and WIFI Module

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B5000

1.Mother Board Specification:

| Model | QY-B5000 |
| --- | --- |
| CPU | i7-1355U |
| Memory[1] | 2*DDR V RAM Slots, Up to 64GB |
| Storage | 1*MSATA SSD Slot 1*M.2 NVME SSD Slot 1*SATA SSD Slot |
| Display | 1*HDMI: Resolution up to 4096*2304@60Hz 1*VGA: Resolution up to 1920*1200@60Hz |
| Expansion | 1*Mini PCIE+1*M.2 Slots Support 4G/5G and WIFI module |
| Ethernet | 2*Intel i225V LAN Chip (10 / 100 / 1000 / 2500 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0+2*USB 2.0 (Type-A) |
| COM | 2*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (DB9 Type) |
| Audio | 1*Line-In+1*Line-Out |
| GPIO | 14 ports GPIO (7*GPI+7*GPO, Phoenix Terminal Type) |

Notes:

- [1]: The maximum memory capacity and the maximum capacity per strip supported by this motherboard depend on the CPU.

QY-B5000

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 3 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 236mm*157mm*72mm |
| System | Windows and Linux |

QY-B5000

3.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5000 | 13th | 2 | 6 | 4*RS-232 2*RS-232/422/485 | 1*HDMI 1*VGA | 2*DDR 5 Up to 96GB | 1*M.2 NVME 1*MSATA 1*SATA | 2*M.2 | DC 9-36V |

4.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - i7-1355U | - 10 core, 12 total threads, 3.70 GHz to 5.00 GHz, 12MB cache | - Max to 96GB for 2*RAM 5 Slots |

| CPU | System |
| --- | --- |
| - 11th/ 12th / 13th | Windows 10 / 11 and Linux |

5.System Information:

QY-B5000

6.Fittings Information:

|  | DDR 5 | 4GB / 8GB / 16GB / 32GB / 48GB |
| --- | --- | --- |
| - Storage | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | M.2 NVME | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| - WIFI Module | 7260AC / AX210 |  |

7.Package List:

| Adapter | 1*12V 7A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | 2 Pin Phoenix Terminal For Power Switch |
| - Terminal | - 16 Pin phoenix terminal for GPIO |

8.Dimensions:

QY-B5000