Industrial Mini PC

QY-B5100

Rail Mount Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Celeron and Core 4/6/7/8/10th-i3 / i5 / i7 CPU

- 3*LAN, 4*USB, 4*COM, 1*HDMI, 1*DP, 8*GPIO

- 1*Mini PCIE Expansion Slot, Support 4G and WIFI Module

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B5100

1.Mother Board Specification:

| Model | QY-B5100 |  |
| --- | --- | --- |
| CPU | J1900 i5-4260U | J6412 i3-7100U i5-6200U / 7200U / 8260U / 10210U i7-7500U / 10610U |
| Memory[1] | 1*DDR III RAM Slot, Up to 8GB | 1*DDR IIII RAM Slot, Up to 16GB |
| Storage | 1*MSATA SSD Slot |  |
| Display[2] | 1*HDMI: Resolution up to 4096*2304@24Hz 1*DP: Resolution up to 4096*2304@60Hz |  |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module |  |
| Ethernet | 3*Intel i211V LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |  |
| USB | 4*USB 3.0 (Type-A) |  |
| COM | 2*RS-232 / 422 / 485 (Phoenix Terminal Type) 2*RS-232 (Phoenix Terminal Type) |  |
| GPIO | 8 ports GPIO (4*GPI+4*GPO, Phoenix Terminal Type) |  |

Notes:

- [1]: The maximum memory capacity and the maximum capacity per strip supported by this motherboard depend on the CPU.

- [2]: The maximum resolution and frame rate of the display interface output depends on the CPU.

QY-B5100

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 3 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall / Rail Mount |
| Size | 260mm*150mm*65mm |
| System | Windows and Linux |

QY-B5100

3.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5100 | J1900 / 4th | 3 | 4 | 2*RS-232/422/485 2*RS-232 | 1*HDMI 1*DP | 1*DDR 3 Up to 8GB | 1*MSATA | 1*Mini PCIE | DC 9-36V |
|  | J6412/6th/7th |  |  |  |  | 1*DDR 4 Up to 16GB |  |  |  |
|  | 8th / 10th |  |  |  |  | 1*DDR 4 Up to 32GB |  |  |  |

4.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - J1900 | - 4 core, 4 total threads, 2.00 GHz to 2.42GHz, 2 MB cache | Max to 8GB for 1*DDR 3 Slot |
| - i5-4260U | - 2 core, 4 total threads, 1.40 GHz to 2.70GHz, 3 MB cache | - Max to 16GB for 1*DDR 3 Slot |
| - J6412 | 4 core, 4 total threads, 2.00 GHz to 2.60GHz, 1.5 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-6200U | 2 core, 4 total threads, 2.30 GHz to 2.80GHz, 3 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i3-7100U | 2 core, 4 total threads, 2.40 GHz, 3 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-7200U | 2 core, 4 total threads, 2.50 GHz to 3.10GHz, 3 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i7-7500U | 2 core, 4 total threads, 2.70 GHz to 3.50GHz, 4 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-8260U | 4 core, 8 total threads, 1.60 GHz to 3.90GHz, 6 MB cache | Max to 32GB for 1*DDR 4 Slot |
| i5-10210U | 4 core, 8 total threads, 1.60 GHz to 4.20GHz, 6 MB cache | Max to 32GB for 1*DDR 4 Slot |
| i7-10610U | 4 core, 8 total threads, 1.80 GHz to 4.90GHz, 6 MB cache | Max to 32GB for 1*DDR 4 Slot |

QY-B5100

5.System Information:

| CPU | System |
| --- | --- |
| - J1900 | Windows 7 / 10 and Linux |
| - 4th / 6th / 7th |  |
| - J6412 | Windows 10 / 11 and Linux |
| - 8th / 10th |  |

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| - Storage | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
| - GSM Module | 4G | 4G+GPS |
| - WIFI Module | 7260AN / 7260AC / AX210 |  |

6.Fittings Information:

7.Package List:

| Adapter | 1*12V 5A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | - 10 Pin phoenix terminal for GPIO |
| - Terminal | - 14 Pin phoenix terminal for COM |

QY-B5100

8.Dimensions: