Industrial Mini PC

QY-B5300-1

Small Size Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Celeron / Atom and Core 4/5/6/7/8/10/12/13th-i3 / i5 / i7 CPU

- 2*LAN, 6*USB, 2*COM, 1*HDMI, 1*VGA

- 1*Mini PCIE Expansion Slot, Support 4G and WIFI Module

- Support Optional Fan Design Chassis

- DC 12V Power Input

- -20℃~60℃ Temperature Can Keep 24/7 Work Stably

QY-B5300-1

1.Mother Board Specification:

| Model | QY-B5300-1 |
| --- | --- |
| CPU | N2840 / J1900 i3-5th i5-4th / 5th i7-4th / 5th |
| Memory | 1*DDR III RAM Slot, Up to 8GB |
| Storage | 1*MSATA SSD Slot 1*SATA SSD Slot |
| Display[1] | 1*HDMI: Resolution up to 3280*2000@60Hz 1*VGA: Resolution up to 1920*1200@60Hz |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module |
| Ethernet | 2*Realtek 8111E LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 1*USB 3.0 (Type-A) 5*USB 2.0 (Type-A) |
| COM | 2*RS-232 (DB9 Type) |
| Audio | 1*Line-Out |

Notes:

- [1]: The maximum resolution and frame rate of the display interface output depends on the CPU.

2.Device Specification:

QY-B5300-1

| Power Input | DC 12V |
| --- | --- |
|  | Support Auto-Start |
|  | 1*2.5mm Circle Type DC Plug |
| Working Temperature | -20℃~60℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
|  | Fan design Chassis (optional) |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 159.6mm*127mm*53mm |
| System | Windows and Linux |

QY-B5300-1

3.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5300-1 | N2840 / J1900 4th / 5th | 2 | 6 | 2*RS-232 | 1*HDMI 1*VGA | 1*DDR 3 Up to 8GB | 1*MSATA 1*SATA | 1*Mini PCIE | DC 12V |
| QY-B5300-2 | J6412 | 2 | 4 | 2*RS-232/485 | 2*HDMI 1*DP | 1*DDR 4 Up to 16GB | 1*MSATA 1*SATA | 2*Mini PCIE |  |
|  | N95 / N100 |  |  |  |  |  | 1*M.2 NVME 1*SATA | 2*M.2 |  |
| QY-B5300-3 | 6th / 7th | 2 | 8 | 2*RS-232 | 1*HDMI 1*VGA | 1*DDR 4 UP to 16GB | 1*M.2 NVME 1*SATA | 1*Mini PCIE |  |
|  | 8th / 10th |  |  |  |  | 2*DDR 4 UP to 32GB |  |  |  |
|  | 12th / 13th |  |  |  |  | 1*DDR 5 Up to 32GB |  |  |  |
| QY-B5300-4E | J6412 | 4 | 6 | 2*RS-232 | 1*HDMI 1*DP | 1*DDR 4 Up to 16GB | 1*M.2 NVME 1*SATA | 1*Mini PCIE 1*M.2 |  |
|  | N100 |  |  |  |  |  | 1*MSATA 1*SATA | 1*M.2 |  |
| QY-B5300-6C | J6412 | 2 | 4 | 2*RS-232/485 4*RS-232 | 2*HDMI 1*DP | 1*DDR 4 Up to 16GB | 1*MSATA 1*SATA | 2*Mini PCIE | DC 9-36V |
|  | N95 / N100 |  |  |  |  |  | 1*M.2 NVME 1*SATA | 2*M.2 |  |

4.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - N2840 | 2 core, 2 total threads, 2.16 GHz to 2.58 GHz, 1 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - J1900 | - 4 core, 4 total threads, 2.00 GHz to 2.42 GHz, 2 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i5-4300U | 2 core, 4 total threads, 1.90 GHz to 2.90 GHz, 3 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i7-4600U | - 2 core, 4 total threads, 2.10 GHz to 3.30 GHz, 4 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i3-5005U | 2 core, 4 total threads, 2.000 GHz, 3 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i5-5300U | - 2 core, 4 total threads, 2.30 GHz to 2.90 GHz, 3 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i7-5600U | - 4 core, 8 total threads, 2.60 GHz to 3.20 GHz, 4 MB cache | - Max to 8GB for 1*RAM 3 slot |

QY-B5300-1

5.System Information:

| CPU | System |
| --- | --- |
| - N2840 / J1900 | Windows 7 / 10 and Linux |
| - 4th / 5th |  |

6.Fittings Information:

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
| - Storage | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
| - WIFI Module | 7260AC / AX210 |  |

QY-B5300-1

7.Package List:

| Adapter | 1*12V 5A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |

8.Fan Design Chassis:

9.Dimensions:

QY-B5300