Industrial Mini PC

QY-B5400

Full IP66 Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Celeron and Core 4/7/8/10th- i5 / i7 CPU

- 2*LAN, 4*USB, 6*COM, 1*VGA, 8*GPIO

- 1*Mini PCIE+1*M.2 Expansion Slots, Support 4G/5G and WIFI Module

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

- Full IP66 Water Proof Protection

QY-B5400

1.Mother Board Specification:

| Model | QY-B5400 |  |
| --- | --- | --- |
| CPU | J1900 i5-4260U | J6412 i5-7200U / 8260U / 10210U i7-7500U / 10710U |
| Memory | 1*DDR III RAM Slot, Up to 8GB | 1*DDR IIII RAM Slot, Up to 16GB |
| Storage | 1*MSATA SSD Slot |  |
| Display[1] | 1*VGA: Resolution up to 1920*1200@60Hz |  |
| Expansion | 1*Mini PCIE Slot+1*M.2 Slot, Support 4G/5G and WIFI module |  |
| Ethernet[2] | 2*Intel i210V LAN Chip (10 / 100 / 1000 Mbps, M12 Type) |  |
| USB[3] | 4*USB 2.0 (M12) |  |
| COM[4] | 2*RS-232 / 422 / 485 (M12 Type) 4*RS-232 (M12 Type) |  |
| GPIO[5] | 8 ports GPIO (4*GPI+4*GPO, M12 Type, optional) |  |

Notes:

[1][2][3][4][5]: All ports will be welded to M12 type: 1*VGA port=1*M12 port, 1*Ethernet port=1*M12 port, 2*USB ports=1*M12 port, 2*COM ports=1*M12 port

Default configuration: 1*Power supply port+2*Ethernet ports+4*USB ports+1*RS-485+1*RS-232 ports+1*VGA port (Total 7*M12 ports), Please inform the sales person of the specific specifications you require if you require interfaces beyond the default configuration.

2.Device Specification:

QY-B5400

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | M12 Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 260mm*152mm*43mm |
| System | Windows and Linux |
| Protection | Full IP66 Water Proof (EN60529:1991+A1:2000+A2:2013) |

QY-B5400

3.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5400 | J1900 / 4th | 2 | 4 | 2*RS-232/422/485 4*RS-232 | 1*VGA | 1*DDR 3 Up to 8GB | 1*MSATA | 1*Mini PCIE 1*M.2 | DC 9-36V |
|  | J6412 / 6th / 7th |  |  |  |  | 1*DDR 4 Up to 16GB |  |  |  |
|  | 8th / 10th |  |  |  |  | 1*DDR 4 Up to 32GB |  |  |  |

4.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - J1900 | - 4 core, 4 total threads, 2.00 GHz to 2.42GHz, 2 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i5-4260U | - 4 core, 4 total threads, 1.40 GHz to 2.70GHz, 3 MB cache | - Max to 8GB for 1*RAM 3 slot |
| - i5-6200U | - 2 core, 4 total threads, 2.30 GHz to 2.80GHz, 3 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i5-7200U | - 2 core, 4 total threads, 2.50 GHz to 3.10 GHz, 3 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i7-7500U | - 2 core, 4 total threads, 2.70 GHz to 3.50 GHz, 4 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i5-8260U | - 4 core, 8 total threads, 1.60 GHz to 3.90 GHz, 6 MB cache | - Max to 32GB for 1*RAM 4 slot |
| - i5-10210U | - 4 core, 8 total threads, 1.60 GHz to 4.20 GHz, 6 MB cache | - Max to 32GB for 1*RAM 4 slot |
| - i7-10710U | - 6 core, 12 total threads, 1.60 GHz to 4.70 GHz, 12MB cache | - Max to 32GB for 1*RAM 4 slot |

QY-B5400

5.System Information:

| CPU | System |
| --- | --- |
| - J1900 | Windows 7 / 10 and Linux |
| - 4th / 6th / 7th |  |
| - J6412 | Windows 10 / 11 and Linux |
| - 8th / 10th |  |

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| - Storage | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
| - GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| - WIFI Module | 7260AN / 7260AC / AX210 |  |
| - CAN BUS Module | 2*CAN BUS interfaces |  |

6.Fittings Information:

7.Package List:

| Adapter | 1*12V 5A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - M12 Cable-1 | - M12 port cable for Ethernet |
| - M12 Cable-2 | - M12 port cable for Ethernet |
| - M12 Cable-3 | - M12 port cable for USB |
| - M12 Cable-4 | - M12 port cable for USB |
| - M12 Cable-5 | - M12 port cable for VGA |
| - M12 cable-6 | - M12 port cable for COM |

QY-B5400

8.Dimensions: