Industrial Mini PC

QY-B5600

Rich Configuration Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Core 6 / 7th-i3 / i5 / i7 CPU

- 5*LAN, 10*USB, 8*COM, 1*HDMI, 1*VGA, 14*GPIO

- 2*Mini PCIE Expansion Slots, Support 4G and WIFI Module

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B5600

1.Mother Board Specification:

| Model | QY-B5600 |
| --- | --- |
| CPU | i3-6157U i5-7200U / 7267U i7-7500U |
| Memory | 1*DDR IIII RAM Slot, Up to 16GB |
| Storage | 1*MSATA SSD Slot 2*SATA SSD Slots |
| Display | 1*HDMI: Resolution up to 4096*2304@60Hz 1*VGA: Resolution up to 1920*1200@60Hz |
| Expansion | 2*Mini PCIE Slots, Support 4G and WIFI module |
| Ethernet | 5*Intel i210V LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.0 (Type-A) 6*USB 2.0 (Type-A) 2*PS/2 |
| COM | 4*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (DB9 Type) |
| GPIO | 14 ports GPIO (7*GPI+7*GPO, Phoenix Terminal Type) |
| Audio | 1*Line-In+1*Line-Out |

QY-B5600

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 3 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 236mm*228.2mm*72.2mm |
| System | Windows and Linux |

QY-B5600

3.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5600 | 6th / 7th | 5 | 10 | 4*RS-232/422/485 4*RS-232 | 1*HDMI 1*VGA | 1*DDR 4 Up to 16GB | 1*MSATA 2*SATA | 2*Mini PCIE | DC 9-36V |

4.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - i3-6157U | - 2 core, 4 total threads, 2.40GHz, 3 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i5-7200U | - 2 core, 4 total threads, 2.50 GHz to 3.10 GHz, 3 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i5-7267U | - 2 core, 4 total threads, 3.10 GHz to 3.50 GHz, 4 MB cache | - Max to 16GB for 1*RAM 4 slot |
| - i7-7500U | - 2 core, 4 total threads, 2.70 GHz to 3.50 GHz, 4 MB cache | - Max to 16GB for 1*RAM 4 slot |

5.System Information:

| CPU | System |
| --- | --- |
| - 6th / 7th | Windows 7 / 10 and Linux |

QY-B5600

6.Fittings Information:

| - RAM | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| --- | --- | --- |
| - Storage | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
| - WIFI Module | 7260AN / 7260AC / AX210 |  |

7.Package List:

| Adapter | 1*12V 7A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | - 2 Pin phoenix terminal for Power Switch |

8.Dimensions:

QY-B5600