Industrial Mini PC

QY-B5700-C

Compound Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel Celeron and Core 7/8/10/11/12th-i3 / i5 / i7 CPU

- 2*LAN, 8*USB, 6*COM, 1*HDMI, 1*VGA, 14*GPIO

- Mini PCIE or M.2 Expansion Slots, Support 4G/5G and WIFI Module

- 1*Multifunctional Expansion Daughter Card

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B5700-C

1.Mother Board Specification:

| Model | QY-B5700-C |  |  |
| --- | --- | --- | --- |
| CPU | J6412 | i3: 7100U / 8130U / 10110U i5: 7200U / 7267U / 8250U / 10310U i7: 7500U / 8550U / 10710U | i3: 1115G4 / 1215U i5: 1135G7 / 1235U i7: 1195G7 / 1255U / 1270P |
| Memory[1] | 2*DDR IIII RAM Slots, Up to 32GB |  |  |
| Storage | 1*MSATA SSD Slot | 1*M.2 NVME SSD Slot |  |
|  | 1*SATA SSD Slot |  |  |
| Display[2] | 1*HDMI: Resolution up to 4096*2304@60Hz 1*VGA: Resolution up to 1920*1200@60Hz |  |  |
| Expansion | 2*Mini PCIE Slots Support 4G/5G and WIFI module |  | 2*M.2 Slots Support 4G/5G and WIFI module |
| Ethernet | 2*Intel i210V LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |  |  |
| USB | 4*USB 3.0 (Type-A) 4*USB 2.0 (Type-A) 1*USB 2.0 (Type-A, Built-in) |  |  |
| COM | 4*RS-232 / 422 / 485 (DB9 Type) 2*RS-232 (DB9 Type) |  |  |
| GPIO | 14 ports GPIO (7*GPI+7*GPO, Phoenix Terminal Type) |  |  |
| Audio | 1*Line-In+1*Line-Out |  |  |

Notes:

- [1]: The maximum memory capacity and the maximum capacity per strip supported by this motherboard depend on the CPU.

- [2]: The maximum resolution and frame rate of the display interface output depends on the CPU.

QY-B5700-C

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 2 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 236mm*183mm*72mm |
| System | Windows and Linux |

QY-B5700-C

3.Daughter Card Specification:

| - COM Port-2 | - 2*RS-232 / 485 (DB9 type) |
| --- | --- |
| - COM Port-6 | - 4*RS-232 / 485+2*RS-232 (Phoenix terminal type) |
| - Ethernet Port-2 | - 2*Intel i211V LAN Chip (RJ-45 type) |
| - Ethernet Port-4 | - 4*Intel i211V LAN Chip, Support POE function (RJ-45 type) |
| - 10G Ethernet Port-2 | - 2*10Gbps Ethernet Ports |
| - USB Port-4 | - 4*USB 3.0 (Type-A) |
| - GPIO Port-14 | - 14 ports GPIO, 7*GPI+7*GPO, 0-3.3V (Phoenix terminal type) |
| - DIDO Port-16 | - 16 ports DIDO, 8*DI+8*DO, 24V (Phoenix terminal type) - Comes with 4*RS-485 COM ports (Phoenix terminal type) |
| - Mini PCIE Slot-2 | - 2*Mini PCIE slots, Support 4G and WIFI module |
| - M.2 Slot-1 | - 1*M.2 Key-B slot, Support 5G module |
| - M.2 NVME SSD Slot-1 | - 1*M.2 NVME SSD slot |
| - SATA SSD Slot-1 | - 1*SATA 3.0 SSD Slot, Support RAID 1 |
| - CAN Port-2 | - 2*CAN interfaces (Phoenix terminal type) |
| - UPS battery | - 1.2AH or 4.8AH |

QY-B5700-C

4.Ordering Information:

| Model | CPU | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B5700-C | J6412 | 2 | 8 | 4*RS-232/422/485 2*RS-232 | 1*HDMI 1*VGA | 2*DDR 4 Up to 32GB | 1*MSATA 1*SATA | 2*Mini PCIE | DC 9-36V |
|  | 7th / 8th |  |  |  |  | 2*DDR 4 Up to 32GB | 1*M.2 NVME 1*SATA | 2*Mini PCIE |  |
|  | 10th |  |  |  |  | 2*DDR 4 Up to 64GB |  |  |  |
|  | 11th / 12th |  |  |  |  | 2*DDR 4 Up to 64GB |  | 2*M.2 |  |

5.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| - J6412 | 4 core, 4 total threads, 2.00 GHz to 2.60GHz, 1.5 MB cache | Max to 32GB for 2*DDR 4 Slots |
| - i3-7100U | - core, 4 total threads, 2.20 GHz to 3.40GHz, 3 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i5-7200U | - 2 core, 4 total threads, 2.50 GHz to 3.10 GHz, 3 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i5-7267U | - 2 core, 4 total threads, 3.10 GHz to 3.50 GHz, 4 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i7-7500U | - 2 core, 4 total threads, 2.70 GHz to 3.50 GHz, 4 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i5-8250U | - 4 core, 8 total threads, 1.60 GHz to 3.40 GHz, 6 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i7-8550U | - 4 core, 8 total threads, 1.80 GHz to 4.00 GHz, 8 MB cache | - Max to 32GB for 2*DDR 4 Slots |
| - i3-10110U | - 2 core, 4 total threads, 2.10 GHz to 4.10 GHz, 4 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i5-10310U | - 4 core, 8 total threads, 1.70 GHz to 4.40 GHz, 6 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i7-10710U | - 6 core, 12 total threads, 1.10 GHz to 4.70 GHz, 12 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i3-1115G4 | - 2 core, 4 total threads, 4.10 GHz, 6 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i5-1135G7 | - 4 core, 8 total threads, 4.20 GHz, 8 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i7-1195G7 | - 4 core, 8 total threads, 2.90 GHz to 5.00 GHz, 12 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i3-1215U | - 6 core, 8 total threads, 3.30 GHz to 4.40 GHz, 10 MB cache | - Max to 64GB for 2*RAM 4 slots |
| - 15-1235U | - 10 core, 12 total threads, 3.30 GHz to 4.40 GHz, 12MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i7-1255U | - 10 core, 12 total threads, 3.50 GHz to 4.70 GHz, 12MB cache | - Max to 64GB for 2*RAM 4 slots |
| - i7-1270P | - 12 core, 16 total threads, 3.50 GHz to 4.80 GHz, 18MB cache | - Max to 64GB for 2*RAM 4 slots |

QY-B5700-C

6.System Information:

| CPU | System |
| --- | --- |
| - 7th | Windows 7 / 10 and Linux |
| - J6412 / 8th / 10th - 11th / 12th | Windows 10 / 11 and Linux |

7.Fittings Information:

| - RAM | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| --- | --- | --- |
| - SSD | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | M.2 NVME | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| - WIFI Module | 7260AC / AX210 |  |
| - CAN BUS Module | 2*CAN BUS Ports |  |

QY-B5700-C

8.Package List:

| Adapter | 1*12V 7A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | - 2 Pin phoenix terminal for Power Switch |
| - Terminal-2 | - 2*8 Pin phoenix terminal for GPIO |

9.Dimensions: