Industrial Mini PC

QY-B6000

Desktop CPU Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel B85 / B365 Chipset

- Intel Celeron, Pentium and Core 4/6/7/8/9th-i3 / i5 / i7 / i9 CPU

- 2*LAN, 8*USB, 6*COM, 1*HDMI, 1*DP, 1*DVI

- 1*Mini PCIE Expansion Slot, Support 4G and WIFI Module

- 1*PCIE 16X or 2*PCI Slots

- DC 9-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B6000

1-1.B85 Mother Board Specification:

| Model | QY-B6000-2E | QY-B6000-4E | QY-B6000-6E |
| --- | --- | --- | --- |
| Chipset | B85 |  |  |
| CPU | Celeron G Series Core 4th-i3 / i5 / i7 |  |  |
| Memory | 2*DDR IIII RAM Slots, Up to 16GB |  |  |
| Storage | 1*MSATA SSD Slot 1*M.2 NVME SSD Slot 2*SATA SSD Slots |  |  |
| Display | 1*HDMI: Resolution up to 4096*2304@24Hz 1*DP: Resolution up to 3840*2160@60Hz 1*DVI: Resolution up to 1920*1200@60Hz |  |  |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module 1*PCIE 16X or 2*PCI |  |  |
| Ethernet | 2*Intel i211V LAN Chip | 4*Intel i211V LAN Chip | 2*Intel i211V+4*Intel i210 LAN Chip (Support POE) |
|  | RJ-45 Type (10 / 100 / 1000 Mbps) |  |  |
| USB | 6*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) |  |  |
| COM | 2*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (DB9 Type) | 4*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (DB9 Type) | 6*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (Phoenix Terminal Type) |
| Audio | 1*Line-In+1*Line-Out |  |  |

QY-B6000

1-2.B365 Mother Board Specification:

| Model | QY-B6000-2E | QY-B6000-4E | QY-B6000-6E |
| --- | --- | --- | --- |
| Chipset | B365 |  |  |
| CPU | Celeron G Series Core 6 / 7 / 8 / 9th-i3 / i5 / i7 / i9 |  |  |
| Memory | 2*DDR IIII RAM Slot, Up to 64GB |  |  |
| Storage | 1*MSATA SSD Slot 1*M.2 NVME SSD Slot 2*SATA SSD Slots |  |  |
| Display | 1*HDMI: Resolution up to 4096*2304@24Hz 1*DP: Resolution up to 3840*2160@60Hz 1*DVI: Resolution up to 1920*1200@60Hz |  |  |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module 1*PCIE 16X or 2*PCI |  |  |
| Ethernet | 2*Intel i225V LAN Chip | 2*Intel i211V+2*Intel i225V LAN Chip | 4*Intel i210V+2*Intel i225V LAN Chip (Support POE) |
|  | RJ-45 Type (10 / 100 / 1000 / 2500 Mbps) |  |  |
| USB | 6*USB 3.0 (Type-A) 2*USB 2.0 (Type-A) |  |  |
| COM | 2*RS-232/422/485 (DB9 Type) 4*RS-232 (DB9 Type) | 4*RS-232 / 422 / 485 (DB9 Type) 4*RS-232 (DB9 Type) | 6*RS-232/422/485 (DB9 Type) 4*RS-232 (Phoenix Terminal Type) |
| Audio | 1*Line-In+1*Line-Out |  |  |

QY-B6000

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 9-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 3 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 290.5mm*232mm*117.5mm |
| System | Windows and Linux |

QY-B6000

3.Ordering Information:

| Model | Chipset | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B6000-2E | B85 | 2 | 8 | 2*RS-232/422/485 4*RS-232 | 1*HDMI 1*DP 1*VGA | 2*DDR 3 Up to 16GB | 1*M.2 NVME 1*MSATA 2*SATA | 1*Mini PCIE PCIE or PCI | DC 9-36V |
| QY-B6000-4E |  | 4 |  | 4*RS-232/422/485 4*RS-232 |  |  |  |  |  |
| QY-B6000-6E |  | 6 |  | 6*RS-232/422/485 4*RS-232 |  |  |  |  |  |
| QY-B6000-2E | B365 | 2 |  | 2*RS-232/422/485 4*RS-232 |  | 2*DDR 4 Up to 64GB |  |  |  |
| QY-B6000-4E |  | 4 |  | 4*RS-232/422/485 4*RS-232 |  |  |  |  |  |
| QY-B6000-6E |  | 6 |  | 6*RS-232/422/485 4*RS-232 |  |  |  |  |  |

4.System Information:

| Chipset | CPU |  |
| --- | --- | --- |
| - B85 | Celeron-G Series | Windows XP / 7 / 10 and Linux |
|  | 4th | Windows 7 / 10 and Linux |
| - B365 | 6th / 7th | Windows 7 / 10 and Linux |
|  | 8th / 9th | Windows 10 / 11 and Linux |
|  | Celeron-G series |  |

5.Fittings Information:

QY-B6000

| - RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| - Storage | M.2 NVME | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
| - WIFI Module | 7260AN / 7260AC / AX210 |  |
| - GPU | NVIDA T600 |  |

6.Package List:

| Adapter | 1*12V 10A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | 2 Pin Phoenix Terminal for Power |

QY-B6000-6E

QY-B6000-4E

7.Picture:

QY-B6000

QY-B6000

8.Dimensions: