Industrial Mini PC

QY-B6100

Desktop CPU Series

- Products Features:

- Industrial-Grade Embedded Fanless Mini PC

- Intel H670 Chipset

- Intel Celeron, Pentium and Core 12/13/14th-i3/i5/i7/i9 CPU

- 2*LAN, 7*USB, 6*COM, 1*HDMI, 1*VGA, 1*DP, 14*GPIO

- 1*Mini PCIE+1*M.2 Expansion Slots, Support 4G/5G and WIFI Module

- 1*PCIE 16X+1*PCIE 1X or 2*PCI Slots

- 2*Multifunctional Expansion Daughter Card

- DC 19-36V Power Input With Overvoltage Protection

- -30℃~70℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-B6100

1.Mother Board Specification:

| Model | QY-B6100 |
| --- | --- |
| Chipset | H670 |
| CPU | Celeron G Series / Pentium Gold Series Core 12 / 13 / 14th-i3 / i5 / i7 / i9 |
| Memory | 2*DDR IIII RAM Slots, Up to 64GB |
| Storage | 1*MSATA SSD Slot 1*M.2 NVME SSD Slot 1*SATA SSD Slots |
| Display | 1*HDMI: Resolution up to 4096*2304@60Hz 1*DP: Resolution up to 4096*2304@60Hz 1*VGA: Resolution up to 1920*1200@60Hz |
| Expansion | 1*Mini PCIE Slot+1*M.2 Slot, Support 4G/5G and WIFI module 1*PCIE 16X+1*PCIE 1X or 2*PCI |
| Ethernet | 2*Intel i211V LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |
| USB | 4*USB 3.1+3*USB 3.0 (Type-A) |
| COM | 4*RS-232 / 422 / 485+2*RS-232 (DB9 Type) |
| Audio | 1*Line-In+1*Line-Out |
| GPIO | 14 ports GPIO (7*GPI+7*GPO, Phoenix terminal type) |
| Daughter Card | 2*Daughter Card Slots，Support expansion I/O interfaces and funcational slots |

QY-B6100

2.Device Specification:

| Watch Dog | Support |
| --- | --- |
| Power Input | DC 19-36V With Overvoltage Protection |
|  | Support Auto-Start |
|  | 4 Pin Phoenix Terminal Type DC Plug |
| Working Temperature | -30℃~70℃, Support 24/7 Working |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fan Design (Fanless design chassis, optional) |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 236mm*220mm*118mm |
| Power Consumption | Max to 240W |
| System | Windows and Linux |

QY-B6100

3.Daughter Card Specification:

| - COM Port-2 | - 2*RS-232 / 485 (DB9 type) |
| --- | --- |
| - COM Port-6 | - 4*RS-232 / 485+2*RS-232 (Phoenix terminal type) |
| - Ethernet Port-2 | - 2*Intel i211V LAN Chip (RJ-45 type) |
| - Ethernet Port-4 | - 4*Intel i211V LAN Chip, Support POE function (RJ-45 type) |
| - 10G Ethernet Port-2 | - 2*10Gbps Ethernet Ports |
| - USB Port-4 | - 4*USB 3.0 (Type-A) |
| - GPIO Port-14 | - 14 ports GPIO, 7*GPI+7*GPO, 0-3.3V (Phoenix terminal type) |
| - DIDO Port-16 | - 16 ports DIDO, 8*DI+8*DO, 24V (Phoenix terminal type) - Comes with 4*RS-485 COM ports (Phoenix terminal type) |
| - Mini PCIE Slot-2 | - 2*Mini PCIE slots, Support 4G and WIFI module |
| - M.2 Slot-1 | - 1*M.2 Key-B slot, Support 5G module |
| - M.2 NVME SSD Slot-1 | - 1*M.2 NVME SSD slot |
| - SATA SSD Slot-1 | - 1*SATA 3.0 SSD Slot, Support RAID 1 |
| - CAN Port-2 | - 2*CAN interfaces (Phoenix terminal type) |
| - UPS battery | - 1.2AH or 4.8AH |

QY-B6100

4.Ordering Information:

| Model | Chipset | LAN | USB | COM | Display | RAM | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-B6100 | H670 | 2 | 7 | 4*RS-232/422/485 2*RS-232 | 1*HDMI 1*DP 1*VGA | 2*DDR 4 | 1*M.2 NVME 1*MSATA 1*SATA | 1*Mini PCIE 1*M.2 PCIE or PCI | DC 19-36V |

5.System Information:

| Chipset | CPU |  |
| --- | --- | --- |
| - H670 | Celeron G Series | Windows 10 / 11 and Linux |
|  | Pentium Gold Series |  |
|  | 12th / 13th / 14th |  |

6.Fittings Information:

| - RAM | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| --- | --- | --- |
| - Storage | M.2 NVME | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| - GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| - WIFI Module | 7260AN / 7260AC / AX210 |  |
| - GPU | Power Consumption＜75W，Size small than 200mm |  |
| - CAN BUS | 2*CAN BUS Ports |  |

QY-B6100

7.Package List:

| Adapter | 1*19V 10A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Terminal | 2 Pin Phoenix Terminal for Power Switch |
| - Terminal | - 8 Pin Phoenix terminal for GPIO port |

8.Dimensions: