Industrial Panel PC

QY-P8133-DC

- Products Features:

- Industrial-Grade Embedded Fan Design Panel PC

- 13.3 inch LED Screen, Resolution: 1920*1080

- Support Capacitive and Resistive Touch Screen

- Intel B365 / H670 Chipset

- Intel Celeron / Pentium and Core 6/7/8/9/12/13/14th-i3 / i5 / i 7 / i9 CPU

- 2*RJ-45, 4*USB, 6*COM, 1*HDMI

- 2* Expansion Slots, Support 4G/5G and WIFI module

- DC 9-36V Power Input With Overvoltage Protection

- -20℃~50℃ Temperature Can Keep 24/7 Work Stably

- 1*Multifunction Daughter card slot

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-P8133-DC

| Model | QY-CFL-B365-8000 | QY-ADL-H670-8000 |
| --- | --- | --- |
| Chipset | B365 | H670 |
| CPU | Celeron Pentium 6 / 7 / 8 / 9th-i3 / i5 / i7 / i9 | Celeron Pentium 12 / 13 / 14th-i3 / i5 / i7 / i9 Desktop CPU |
| Memory | 2*DDR IIII RAM Slots, Up to 64GB | 2*DDR IIII RAM Slots, Up to 128GB |
| Storage | 1*M.2 NVME SSD Slot |  |
| Display | 1*HDMI: Resolution up to 4096*2106@24Hz |  |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module | 2*M.2 Slots, Support 4G/5G and WIFI module |
| Ethernet | 2*Intel LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |  |
| USB | 4*USB 3.0 (Type-A) |  |
| COM | 2*RS-232 / 422 / 485 (DB9 type) 4*RS-232 (DB9 ype) |  |
| Audio | 1*Line-Out |  |
| Daughter Card | Support expansion more I/O interfaces and functional slots |  |

1. Mother Board Specification:

QY-P8133-DC

2.Screen Specification:

| Screen Size | 13.3 inch TFT LED Screen, 16:9 Size |  |
| --- | --- | --- |
| Resolution | 1920*1080 |  |
| Brightness | 350nits |  |
| Contrast | 800:1 |  |
| Largest Colorful | 16M |  |
| Touch Screen Type | Capacitive | Resistive |
| Transmittance | More than 95% | More than 95% |
| Touch Life | 50 million times | 35 million time |
| Response Time | - ＜5ms | - ＜5ms |
| Touch Hardness | 7 Mohs |  |
| Protection | Front Panel IP65 Water Proof |  |

QY-P8133-DC

3.Device Specification:

| Power Input | DC 9-36V with overvoltage protection |
| --- | --- |
|  | Support Auto-Start |
|  | 2 Pin Phoenix Terminal type DC Plug |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fan Design |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 343.5mm*214mm*68mm |
| System | Windows and Linux |

QY-P8133-DC

4.Optional Specification:

| Screen | Brightness | Optional 1000 nits LED screen |
| --- | --- | --- |
|  |  | Optional brightness auto adjustment function |
|  | Anti-Glare | Optional |
|  | Camera | Optional 200w or 500w camera (Built-in front panel) |
|  | Full Lamination | Optional |

QY-P8133-DC

5.Daughter Card Specification:

| - COM Port-2 | - 2*RS-232 / 485 (DB9 type) |
| --- | --- |
| - COM Port-6 | - 4*RS-232 / 485+2*RS-232 (Phoenix terminal type) |
| - Ethernet Port-2 | - 2*Intel i211V LAN Chip (RJ-45 type) |
| - Ethernet Port-4 | - 4*Intel i211V LAN Chip, Support POE function (RJ-45 type) |
| - 10G Ethernet Port-2 | - 2*10Gbps Ethernet Ports |
| - USB Port-4 | - 4*USB 3.0 (Type-A) |
| - GPIO Port-14 | - 14 ports GPIO, 7*GPI+7*GPO, 0-3.3V (Phoenix terminal type) |
| - DIDO Port-16 | - 16 ports DIDO, 8*DI+8*DO, 24V (Phoenix terminal type) - Comes with 4*RS-485 COM ports (Phoenix terminal type) |
| - Mini PCIE Slot-2 | - 2*Mini PCIE slots, Support 4G and WIFI module |
| - M.2 Slot-1 | - 1*M.2 Key-B slot, Support 5G module |
| - M.2 NVME SSD Slot-1 | - 1*M.2 NVME SSD slot |
| - SATA SSD Slot-1 | - 1*SATA 3.0 SSD Slot, Support RAID 1 |
| - CAN Port-2 | - 2*CAN interfaces (Phoenix terminal type) |
| - UPS battery | - 1.2AH or 4.8AH |

QY-P8133-DC

6.Ordering Information:

| Model | CPU | LAN | USB | COM | RAM | Display | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-P8133-DC | B365 | 2 | 4 | 2*RS-232/422/485 4*RS-232 | 2*DDR 4 Up to 64GB | 1*HDMI | 1*M.2 NVME | 1*Mini PCIE | DC 9-36V |
|  | H670 |  |  |  | 2*DDR 4 Up to 128GB |  |  | 2*M.2 |  |

7.System Information:

| CPU | System |
| --- | --- |
| 6th / 7th | Windows 7 / 10 and Linux |
| 8th / 9th / 12th / 13th / 14th | Windows 10 / 11 and Linux |

8.Fittings Information:

| RAM | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| --- | --- | --- |
| SSD | M.2 NVME | 128GB / 256GB / 512GB / 1TB / 2TB / 4TB |
|  | SATA | 120GB / 128GB / 240GB / 256GB / 480GB / 512GB / 960GB / 1TB / 2TB / 4TB |
| GSM Module | 4G | 4G+GPS |
|  | 5G | 5G+GPS |
| WIFI Module | 7260AC / AX210 |  |
| CAN BUS Module | 2 ports CAN BUS interfaces |  |

9.Package List:

| Adapter | 1*12V 10A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Mount Clamp | 4*Mount Clamp |

QY-P8133-DC

10.Dimensions: