Industrial Panel PC

QY-P8101

- Products Features:

- Industrial-Grade Embedded Fanless Panel PC

- 10.1 inch LED Screen, Resolution: 1280*800

- Support Capacitive and Resistive Touch Screen

- Intel Celeron and Core 4/6/7/8/10th-i3/i5/i7 CPU

- 2*RJ-45, 4*USB, 6*COM

- 1*Mini PCIE Expansion Slot, Support 4G and WIFI module

- DC 12V Power Input With Overvoltage Protection

- -20℃~50℃ Temperature Can Keep 24/7 Work Stably

- Anti-Static, Anti-Pulse, Anti-Surge, Anti-Radiation

QY-P8101

1.Mother Board Specification:

| Model | QY-P8101 |  |
| --- | --- | --- |
| CPU | J1900 i5-4260U | N95 / N100 / J6412 i3: 7100U i5: 6200U / 7200U / 8260U / 10210U i7: 7500U / 10610U |
| Memory | 1*DDR III RAM Slot, Up to 8GB | 1*DDR IIII RAM Slot, Up to 16GB |
| Storage | 1*MSATA SSD Slot |  |
| Expansion | 1*Mini PCIE Slot, Support 4G and WIFI module |  |
| Ethernet | 2*Intel i210v LAN Chip (10 / 100 / 1000 Mbps, RJ-45 Type) |  |
| USB | 4*USB 3.0 (Type-A) |  |
| COM | 2*RS-232 / 485 (DB9 type) 1*RS-232 / 485+4*RS-232 (Phoenix Terminal type) |  |
| Audio | 1*Line-Out |  |

2.Screen Specification:

QY-P8101

| Screen Size | 10.1 inch TFT LED Screen, 16:10 Size |  |
| --- | --- | --- |
| Resolution | 1280*800 |  |
| Brightness | 350nits |  |
| Contrast | 800:1 |  |
| Largest Colorful | 16M |  |
| Touch Screen Type | Capacitive | Resistive |
| Transmittance | More than 95% | More than 95% |
| Touch Life | 50 million times | 35 million time |
| Response Time | - ＜5ms | - ＜5ms |
| Touch Hardness | 6 Mohs |  |
| Protection | Front Panel IP65 Water Proof |  |

QY-P8101

3.Device Specification:

| Power Input | DC 12V with overvoltage protection (Support optional DC 9-36V Wide Voltage Power Module) |
| --- | --- |
|  | Support Auto-Start |
|  | 2 Pin Phoenix Terminal type DC Plug |
| Structure | Fully Enclosed Aluminum Alloy Material |
| Heat Dissipation | Fanless Design, Conduction Heat Dissipation |
| Installation | Desktop / Embedded / Wall Mount |
| Size | 258mm*172mm*53.5mm |
| System | Windows and Linux |

QY-P8101

4.Optional Specification:

| Screen | Brightness | Optional 1000 nits LED screen |
| --- | --- | --- |
|  | Anti-Glare | Optional |
|  | Full Lamination | Optional |

QY-P8101

5.Ordering Information:

| Model | CPU | LAN | USB | COM | RAM | Display | SSD | Expansion | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QY-P8101 | J1900 | 2 | 4 | 2*RS-232/485 4*RS-232 | 1*DDR 3 Up to 8GB | None | 1*MSATA | 1*Mini PCIE | DC 12V |
|  | 4th |  |  |  |  |  |  |  |  |
|  | 6th / 7th 8th / 10th |  |  |  | 1*DDR 4 Up to 16GB |  |  |  |  |
|  | N95 / N100 |  |  |  |  |  |  |  | DC 9-36V |
|  | J6412 |  |  |  |  | 1*HDMI |  | 1*Mini PCIE 1*M.2 | DC 12V |

6.Package List:

| Adapter | 1*12V 5A Adapter |
| --- | --- |
| - Power Cable | 1*Power Cable |
| - Mount Clamp | 4*Mount Clamp |

QY-P8101

7.CPU Information:

| CPU | Specification |  |
| --- | --- | --- |
| J1900 | 4 core, 4 total threads, 2.00 GHz to 2.42GHz, 2 MB cache | Max to 8GB for 1*DDR 3 Slot |
| i5-4260U | - 2 core, 4 total threads, 1.40 GHz to 2.70GHz, 3 MB cache | - Max to 8GB for 1*DDR 3 Slot |
| J6412 | 4 core, 4 total threads, 2.00 GHz to 2.60GHz, 1.5 MB cache | Max to 16GB for 1*DDR 4 Slot |
| N100 | 4 core, 4 total threads, 3.40 GHz, 6 MB cache | Max to 16GB for 1*DDR 4 Slot |
| J6412 | 4 core, 4 total threads, 2.00 GHz to 2.60GHz, 1.5 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-6200U | 2 core, 4 total threads, 2.30 GHz to 2.80GHz, 3 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i3-7100U | 2 core, 4 total threads, 2.40 GHz, 3MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-7200U | 2 core, 4 total threads, 2.50 GHz to 3.10GHz, 3 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i7-7500U | 2 core, 4 total threads, 2.70 GHz to 3.50GHz, 4 MB cache | Max to 16GB for 1*DDR 4 Slot |
| i5-8260U | - 4 core, 8 total threads, 1.60 GHz to 3.90 GHz, 6MB cache | Max to 32GB for 1*DDR 4 Slot |
| i5-10210U | - 4 core, 8 total threads, 1.60 GHz to 4.20 GHz, 6MB cache | Max to 32GB for 1*DDR 4 Slot |
| i7-10610U | - 4 core, 8 total threads, 1.80 GHz to 4.90 GHz, 8MB cache | Max to 32GB for 1*DDR 4 Slot |

8.System Information:

| CPU | System |
| --- | --- |
| J1900 | Windows 7 / 10 and Linux |
| 4th / 6th / 7th |  |
| N95 / N100 / J6412 | Windows 10 /11 and Linux |
| 8th / 10th |  |

9.Fittings Information:

QY-P8101

| RAM | DDR 3 | 4GB / 8GB |
| --- | --- | --- |
|  | DDR 4 | 4GB / 8GB / 16GB / 32GB |
| SSD | MSATA | 64GB / 128GB / 256GB / 512GB / 1TB |
| GSM Module | 4G | 4G+GPS |
| WIFI Module | 7260AC / AX210 |  |

10.Dimensions: